1065 South Rogers Circle • Boca Raton, FL 33487 • Phone (561) 998-9600 • Fax (561) 998-9609

Thermal Print Head Cleaning Procedure

The print head should be cleaned periodically to prevent debris from building up on the print element.  The required cleaning interval varies greatly depending on the quality of the ticket stock and the amount of dust entering the print area.  Excessive dirt build up on the print head will result in reduced quality.  Continuing to run the print head in a dirty condition will reduce its life expectancy as it is unable to diffuse its heat properly.

The thermal print head can be removed for cleaning as follows:

- Make sure power is off and the AC cord is disconnected from the printer.

- DO NOT UNPLUG CABLE FROM PRINT HEAD.

- Lift up on the cam lock assembly (located above the head mounting block) to remove pressure from the thermal head. (see photo A)

- Lift up on the head mounting block/thermal head to remove. (see photo B)

- Clean the thermal print head surface (the side that makes contact with the paper) with 
isopropyl alcohol. (see Photo C) Clean the area highlighted in yellow.

- Install the head by reversing the above procedures.

- Restore pressure to the head by pushing down on the cam lock assembly.

- The printer in now ready for operation.  If the print quality is still poor then the thermal head most likely needs to be replaced.

Cam lock

assembly

# If you have any questions please contact us at 561 998 9600 ext. 170